ISA

|  | Opcode[31:24] |  |  |  |  | Immediate [15:0] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operation Name | Operation Type [31] (0 for R type & 1 for J type) | Operation [30:27] | Function [26:24] | R1 [23:20] (Destination reg always) | R2 [19:16] (Source reg always) | R3 [15:12] (Src reg in case of R) |  |
| ADD | 0 | 0000 | 010 |  |  |  | 00…0 |
| SUB | 0 | 0000 | 011 |  |  |  | 00…0 |
| AND | 0 | 0001 | 000 |  |  |  | 00…0 |
| OR | 0 | 0001 | 001 |  |  |  | 00…0 |
| XOR | 0 | 0001 | 011 |  |  |  | 00…0 |
| NOR | 0 | 0001 | 010 |  |  |  | 00…0 |
| NOT | 0 | 0001 | 100 |  |  | 0000 | 00…0 |
| SL | 0 | 0010 | 100 |  |  |  | 00…0 |
| SRL | 0 | 0010 | 000 |  |  |  | 00…0 |
| SRA | 0 | 0010 | 001 |  |  |  | 00…0 |
| INC | 0 | 0000 | 000 |  |  | 0000 | 00…0 |
| DEC | 0 | 0000 | 001 |  |  | 0000 | 00…0 |
| SLT | 0 | 0011 | 000 |  |  |  | 00…0 |
| SGT | 0 | 0011 | 001 |  |  |  | 00…0 |
| HAM | 0 | 0010 | 011 |  |  | 0000 | 00…0 |
| MOVE | 0 | 0100 | 000 |  |  | 0000 | 00…0 |
| CMOV | 0 | 0100 | 001 |  |  |  | 00…0 |
| HALT | 0 | 1111 | 000 | 0000 | 0000 | 0000 | 00…0 |
| NOP | 0 | 1110 | 000 | 0000 | 0000 | 0000 | 00…0 |
| ADDI | 1 | 0000 | 010 |  |  |  |  |
| SUBI | 1 | 0000 | 011 |  |  |  |  |
| ANDI | 1 | 0001 | 000 |  |  |  |  |
| ORI | 1 | 0001 | 001 |  |  |  |  |
| XORI | 1 | 0001 | 011 |  |  |  |  |
| NORI | 1 | 0001 | 010 |  |  |  |  |
| SLI | 1 | 0010 | 100 |  |  |  |  |
| SRLI | 1 | 0010 | 000 |  |  |  |  |
| SRAI | 1 | 0010 | 001 |  |  |  |  |
| SLTI | 1 | 0011 | 000 |  |  |  |  |
| SGTI | 1 | 0011 | 001 |  |  |  |  |
| LUI | 1 | 0010 | 011 |  | 0000 |  |  |
| LD | 1 | 1000 | 000 |  |  |  |  |
| ST | 1 | 1000 | 001 |  |  |  |  |
| BR | 1 | 0101 | 000 | 0000 |  |  |  |
| BMI | 1 | 0101 | 001 | 0000 |  |  |  |
| BPL | 1 | 0101 | 010 | 0000 |  |  |  |
| BZ | 1 | 0101 | 011 | 0000 |  |  |  |

Design Elements

- Register Bank [R0 to R15]

- Custom ALU

- Custom CU

- PC (32 bit reg)

- Stack Pointer (32 bit reg)  (It is R15)

- Memory (Implemented as a linear array of regs). Memory is byte addressable with a 32-bit memory address. All operations are on 32-bit data, and all loads and stores occur from memory addresses that are multiples of 8.

- IR

- MUXs

- SE1 (Sign Extender 1) which will append 16 sign bits on left of immediate data

- SE2 (Sign Extender 2, for branch type instructions, LD and ST) which will append 14 sign bits on left of immediate data and two 0 bits on right (since all memory access (thus, the values of PC as well) has to be a multiple of 4).